9.  RC PHASE SHIFT OSCILLATOR

AIM: To find the frequency of oscillations of the RC phase  Shift oscillator and to measure the phase shift of each  Section of the RC network.

APPARATUS

- Transistor BC 107,

- Resistors – 4.7KΩ (2), 33KΩ (1),2.2KΩ (1),

8.2KΩ (1) and 2.7KΩ (1).

- Capacitors – 0.01μf (3) & 47μf(1),

- Potentiometer 10KΩ (1),

- Regulated Power Supply,

- CRO with probes,    7. Bread Board & wires.

CIRCUIT DIAGRAM 	:

# RC PHASE SHIFT OSCILLATOR

PROCEDURE 	:

- Connections are made as per the circuit diagram.

- Set the value of Rc (4KΩ – 8KΩ) by varying DRB and observe the output waveform at ‘0’ on CRO which is sinusoidal.

- Now, the CRO probe is changed to position ‘B’ such that the output                                                                       Waveform at B is observed on CRO which is shifted by 60o w.r.t ‘0’.

- The output waveform at ‘C’ is observed on CRO, which is shifted by                                                      120 o w.r.t ‘0’.

5 . The output waveform at ‘D’ is observed on CRO, which is shifted by 180o w.r.t ‘0’.

- Theoretically the frequency of oscillations is calculated by the formula,                                         f=1/2חRC√6+4K,       K = RC / R

Practically the time period ‘T’ on CRO is noted and frequency f = 1/T is             calculated.

- The readings for different values of RC at  4K,5K,6K,7K and 8K are noted                                                                                   And are tabulated as shown in the tabular form for different Lissajous            pattern.

- A graph is plotted for phase and amplitude locating the phase shift           observed On CRO at different positions of  (B,C,D).

OBSERVATION :

CIRCUIT DIAGRAM

WAVEFORMS

POSITION B W.R.T COLLECTOR

POSITION C W.R.T COLLECTOR

POSITION D W.R.T COLLECTOR

CALCULATION :

K=RC/R= 220/330 = 2/3=0.67

R=330ohm, C=0.1𝜇F

f= 1/ 2𝜋𝑅𝐶√6+4𝐾 = 1 /2𝜋330×0.1×10−6×√6+4𝐾 =1.6387KHz

T=1/f=610.23𝜇𝑠

(i)Y1=6, Y2=6

θ = sin−1 ( 𝑌1/ 𝑌2 )=90°

(ii)Y1=7, Y2=8

θ = sin−1 ( 𝑌1/ 𝑌2 )=61.04°

(iii)Y1=6, Y2=12

θ = sin−1 ( 𝑌1/ 𝑌2 )=30°

TABULAR FORM 	:

| S.NO | RC (KΩ) | Position w.r.t Collector | Lissajous Pattern | Y1 (V) | Y2 (V) | θ=Sin-1 (Y1 /Y 2) | T (Sec) | f0 (Hz) Theoretical |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4.7KΩ | B C D | Φ =0 0< Φ<90 90 < Φ <180 | 6 7 6 | 6 8 12 | 90 61.04 30 | 610.23 us 610.23 us 610.23 us | 1.6387 KHz 1.6387 KHz 1.6387 KHz |

LISSAZEOUS PATTERN:

Φ =0 0                          0< Φ<900 	 	Φ=900 	 	 	900< Φ<1800 	 	    Φ=1800

x

1

x

2

x

1

x

2

X

Y

Major Axis

x

1

x

2

x

1

x

2

X

Y

Major Axis

MODEL GRAPH:   OUTPUT WAVEFORMS

at  B

at O

at C

at O

at O

at D

Amp(V)

Amp(V)

Amp(V)

Time(sec)

Time(sec)

Time(sec)

∆t

∆t

∆t

at  B

at O

at C

at O

at O

at D

Amp(V)

Amp(V)

Amp(V)

Time(sec)

Time(sec)

Time(sec)

∆t

∆t

∆t

POSITION B W.R.T COLLECTOR

POSITION C W.R.T COLLECTOR

POSITION D W.R.T COLLECTOR

PRECAUTIONS:  1. The readings are to be noted down without parallax error.

2. Wrong connections should be avoided.

RESULT:

Frequency of oscillations of the RC phase  Shift oscillator and the phase shift of each  Section of the RC network has been studied and calculated through the designed circuit.